DIVISION 27 21 29

IEEE 802.3AT GIGABIT PoE+ INJECTOR

CNGE1IPS[75,95]AC

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- ComNet CNGE1IPS[75,95]AC IEEE 802.3at Gigabit PoE+ Injector (Mid-Span)

1.02 SECTION INCLUDES

- CNGE1IPS75AC IEEE 802.3at 75W PoE Midspan Injector

- CNGE1IPS95AC IEEE 802.3at 95W PoE Midspan Injector

1.03 REFERENCES

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provides 1 10/100/1000Base-T copper port with IEEE 802.3at Power over Ethernet injector capability.

- The system shall utilize EIA568, category 5/5e/6, 4-pair cables for 10Base-T, 100Base-TX or 1000Base-T to transfer data and 75W or 95W PoE.

1.05 SUBMITTALS

- Product Data: Manufacturer’s printed product data sheet

- Warranty: Manufacturer’s Printed Warranty

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -40˚ C to +85˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of –40˚ C to +75˚ C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 0% to 95% (non-condensing). If product is installed in condensation conditions, unit shall have conformal coating applied to the printed circuit board.

1.08 WARRANTY

- Standard Communication Networks Comprehensive Lifetime Warranty: Comnet warrants the product to be free of factory defects under manufacture’s Lifetime Warranty as submitted under article 1.05 (E)

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer: Communication Networks 3 Corporate Drive Danbury, CT 06810 USA; Telephone: 203-796-5300; Fax 203-796-5303; Email: sales@comnet.net; Internet: www.comnet.net

- Substitutions: Not Permitted

- All fiber optic modules shall be supplied from a single manufacturer.

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

2.03 GENERAL SPECIFICATIONS

- The Power over Ethernet Injector shall be a Comnet CNGE1IPS[75,95]AC module.

- The module shall comply with IEEE 802.3at Power over Ethernet standard.

- The module shall support the Ethernet data IEEE 802.3/802.3u/802.3ab 10/100/1000Base-T protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The module features two fixed 10/100/1000T electrical ports in compact housing.

- The port printed with “Ethernet” functions as "Data input".

- The other port on the other side and printed with “Ethernet + DC” functions as "PoE (Data and Power) output".

- The module also injects power by transmitting DC Voltage to the Cat5/5e/6 cable and transfer data and power simultaneously between the PSE and PD.

- The module shall Auto-detect PoE IEEE 802.3at equipment and shall protect devices from being damaged by incorrect installation.

- The module shall support total distance up to 100 meters.

- The module shall provide power and PoE in-use status LED indicators for the proper monitoring of operation.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3

- Data Rate: 10/100 /1000 Mbps

- Data Inputs: 2

2.05 STATUS INDICATORS

A. LED Definition:

| LED | Color | Function |
| --- | --- | --- |
| Power | Green | Indicates that the Module has power. |
| Active | Green | Indicates that the port is providing inline power |

2.06 CONNECTORS

- Power: IEC60320 – C13 outlet

- Chassis Ground

- Data:

- Ethernet Port: RJ-45

- Ethernet + Power Port: RJ-45

2.07 ELECTRICAL SPECIFICATIONS

- Power Adapter:

- Input Voltage Range: 100~240VAC

- Output Voltage: 50-57VDC

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions (WxDxH): CNGE1IPS75AZ: 8.1 × 5.1 × 1.6 in (20..4 × 13.0 × 4.1 cm)

- Surface Mount Dimensions (WxDxH): CNGE1IPS95AC:  8.4 × 5.3 × 1.6 in (21.4 × 13.4 × 4.1 cm)

- IP 20 Black powder-coat aluminum

- Weight < 4 lb / 1.84kg

2.9 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >100,000 Hours

- Operating Temp: –40˚ C to +75˚ C

- Storage Temp: -40˚ C to +85˚ C

- Relative Humidity: 0% to 95% (non-condensing). If product is installed under condensation conditions, unit shall have conformal coating applied to the printed circuit board. (Add –C to model number for conformal coated printed circuit board)

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class B

2.11 ACCESSORIES

- AC-to-DC 56V Power Adapter x1 (not included)

- Power cord x 1 (UK ,AU, AG, IR and SW only)

PART 3 – EXECUTION

3.01 EXAMINATION

- Inspect modules before installation.

- Modules shall be free of any cosmetic defects or damage.

- All optical connectors shall be covered with dust caps and remain on the module until installing cable connectors to module.

- Shipping box shall include the module, power supply and operations manual.

3.02 PREPARATION

- Standalone Module (Wall Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

3.03 INSTALLATION

- General: Locate PoE Injector module as indicated on the approved detail drawings and install module in compliance with the ComNet installation manual.

3.04 TESTING

- Testing the 10/100/1000T Gigabit Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the PoE Injector module or other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100/1000T PoE Copper output capability.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| CNGE1IPS75AC | IEEE 802.3AT GIGABIT 75W MID-SPAN POE+ INJECTOR | 300 feet (100m) electrical |
| CNGE1IPS95AC | IEEE 802.3AT GIGABIT 75W MID-SPAN POE+ INJECTOR | 300 feet (100m) electrical |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

## END OF SECTION