DIVISION 27 21 29

CNGE2MC SERIES – FIBER OPTIC GIGABIT ETHERNET TRANSCEIVER

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- Fiber Optic Ethernet Transceiver

1.02 SECTION INCLUDES

- CNGE2MC Series Gigabit Ethernet Transceiver

1.03 REFERENCES

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide an Ethernet transceiver system that transmits bi-directional 10, 100, or 1000Mps data over multimode or single mode fiber connections.

- The system shall utilize CNGE2MC media converter package along with an SFP modules capable of bi-directional data transmission of 10, 100, or 1000Mbps on one or two optical fibers, depending on the SFP module selected. (See Table C, page 3.)

1.05 SUBMITTALS

- Product Data: Manufacturer’s printed product data sheet for each type of Transmitter/Receiver specified.

- Detail Drawings: Electrical and optical connect drawings. Product mounting template.

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for each type of Transmitter/Receiver specified.

- Warranty: Manufacturer’s Printed Warranty

1.06 DELIVERY, STORAGE AND HANDLING

- Deliver materials in unopened factory packaging with Manufacturer’s bar coding to the job site.

- Inspect product upon delivery to assure that specified products have been received.

- Store in original packaging in a climate controlled environment. Storage Temperature not to exceed: -40˚ C to +85˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of –40˚ C to +75˚ C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 0% to 95% (non-condensing). If product is installed in condensation conditions, unit shall have conformal coating applied to the printed circuit board.

1.08 WARRANTY

- Standard Communication Networks Comprehensive Lifetime Warranty: Comnet warrants the product to be free of factory defects under manufacture’s Lifetime Warranty as submitted under article 1.05 (E)

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer: Communication Networks 3 Corporate Drive Danbury, CT USA; Telephone: 203-796-5300; Fax 203-796-5303; Email: sales@comnet.net; Internet: www.comnet.net

- Substitutions: Not Permitted

- All fiber optic modules shall be supplied from a single manufacturer.

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table C: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Ethernet Gigabit data transceiver system shall consist of a Comnet CNGE2MC series media converter and a Comnet SFP module.  The module shall support the transmission of 10, 100, or 1000 Mbps employing singlemode or multimode optical fiber. The module shall support the Ethernet data interface and be IEEE 802.3 compliant.  The unit is set to auto-negotiate data rates and can be used for half duplex or full duplex transmission.  The module shall require no in-field optical adjustments or in-line attenuators to ease installation.  The module shall provide power and fiber link status indicating LED’s for monitoring proper system operation. The modules shall provide automatic re-settable solid-state current limiters and independent voltage regulators on each module to reduce the chance of a single point failure of the system. The module shall have an MTBF of >100,000 hours and operate in an environment of –40˚ C to +75˚ C and relative humidity between  0% to 95% (non-condensing). The module shall be UL and ULC listed and CE marked. The circuit board shall be UL 94 flame rated and meet all PCI standards.  All PC boards shall be designated with part number, PC board number and show appropriate revision number. Housing shall be of all metal construction.  All LED indicators and both electrical and mechanical connections shall be identified with silk-screened labels. The module shall have a lifetime warranty to reduce system life cycle cost in an event of a module failure.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3

- Data Rate: 10/100/1000 Mbps

- Operation Mode: Full or Half Duplex

2.05 OPTICAL SPECIFICATIONS

- Determined by SFP selection – see Table C, page 3.

2.06 STATUS INDICATORS

- Power: On/Green – Off/Off

- Fiber Link:  Fiber connection/Green – No Fiber Connection/Red

2.07 CONNECTORS

- Determined by SFP module selected – see Table C, page 3.

- Power: Screw Clamp Terminal Strip.

- Data: RJ-45

2.08 ELECTRICAL SPECIFICATIONS

- Power: Surface Mount:

12VDC @ 200 mA

- Current Protection: Automatic re-settable solid-state current limiters

- Voltage Regulation: Solid-state, Independent on each board

- Circuit Board: UL 94 flame rated and meets all IPC standards.

2.09 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 6.1” x 5.3” x 1.1” (15.5 cm x 13.5 cm x 2.8 cm)

- Finish: Module shall be constructed of a metal enclosure with all connections and indicators silk-screened directly on unit.

- Weight: <2.0 lbs./0.9kg

2.10 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >100,000 Hours

- Operating Temp: –40˚ C to +75˚ C

- Storage Temp: -40˚ C to +85˚ C

- Relative Humidity: 0% to 95% (non-condensing). If product is installed under condensation conditions, unit shall have conformal coating applied to the printed circuit board. (Add –C to model number for conformal coated printed circuit board)

2.11 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- UL 94-flame rated PCB board

- European Listed (CE)

PART 3 - EXECUTION

3.01 EXAMINATION

- Inspect modules before installation.

- Modules shall be free of any cosmetic defects or damage.

- All optical connectors shall be covered with dust caps and remain on the module until installing cable connectors to module.

- Shipping box shall include the module, power supply and operations manual.

3.02 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the ComNet mounting template and installation manual.

- Optical Fibers

- Caution: NEVER look into the end of an active optical fiber when using laser light output. Eye damage can occur. Wear eye protection when cleaving, terminating, and splicing fiber.

- The number and type (multimode or single-mode) of optical fiber shall meet the requirements of the ComNet model number in article 2.05 used in the installation.

- All optical fiber cables shall be properly installed and terminated with the mating optical connectors as submitted in article 2.07 (A).

- The optical link shall be tested with either a power meter, at a minimum, or OTDR to ensure the link budget (overall path loss) plus an added 3dB of optical safety margin does not exceed the optical power budget as submitted in article 2.05.

- All optical connectors on cable shall be cleaned in compliance to optical connector manufactures specifications and covered with dust caps until connection to the fiber optic module.

3.03 INSTALLATION

- General: Locate fiber optic modules as indicated on the approved detail drawings and install module in compliance with the ComNet installation and operations manual.

3.04 TESTING

- Testing the Fiber Optic Ethernet Link.

- Verify that the data leads and optical fibers are properly connected.

- Make sure that power is applied to all fiber optic modules, controllers, and receiver drivers or other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all fiber scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| CNGE2MC SERIES | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| CNGE2MC | 10/100/1000 Mbps Ethernet Transceiver | SFP-dependant (see Table C) |

Table B: Product Compatibility Chart

| COMPATIBLE TRANSCEIVER | DESCRIPTION |
| --- | --- |
| CNGE2MC | 10/100/1000 Mbps Ethernet Transceiver, Standard Size |
| CNGE2MC-M | 10/100/1000 Mbps Ethernet Transceiver, Minature Size |
| CNGE22MC | Dual 10/100/1000 Mbps Ethernet Transceiver, Standard Size |
| CNGE4US | 4-port 10/100/1000 Mbps Unmanaged Switch |
| CNGE8US | 8-port 10/100/1000 Mbps Unmanaged Switch |
| CNGE3FE7MS | 10-port Managed Switch, (7) 10/100 RJ-45 ports, (3) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |
| CNGE3FE7MS-2 | 10-port Managed Switch, (7) 10/100 RJ-45 ports, (3) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |
| CNGE2FE24MS | 26-port Managed Switch, (24) 10/100 RJ-45 ports, (2) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |
| CNGE2FE24MS-POE | 26-port Managed Switch, (24) 10/100 RJ-45 802.3af PoE ports, (2) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |

Table C: SFP Module Compatibility Chart

| SFP | DESCRIPTION |
| --- | --- |
| SFP-6 | Single Mode, 1000fx, 1310nm, 15km, 2 Fiber, LC |
| SFP-7 | Single Mode, 1000fx, 1310nm, 40km, 2 Fiber, LC |
| SFP-8 | Single Mode, 1000fx,1550nm, 70km, 2 Fiber, LC |
| SFP-9 | Single Mode, 1000fx,1550nm, 120km, 2 Fiber, LC |
| SFP-12 | Single Mode, 1000fx, 1310nm, 20km, 1 Fiber, LC |
| SFP-13 | Single Mode, 1000fx, 1550nm, 20km, 1 Fiber, LC |
| SFP-14 | Single Mode, 1000fx, 1310nm, 20km, 1 Fiber, SC |
| SFP-15 | Single Mode, 1000fx, 1550nm, 20km, 1 Fiber, SC |
| SFP-16 | Multimode, 1000fx, 850 nm 550m, 2 Fiber, LC |
| SFP-17 | Single Mode, 1000fx, 1310nm, 60km, 1 Fiber, LC |
| SFP-18 | Single Mode, 1000fx,1550nm,60km, 1 Fiber, LC |
| SFP-21 | Single Mode, 1000fx, 1310nm, 60km, 1 Fiber, SC |
| SFP-22 | Single Mode, 1000fx,1550nm,60km, 1 Fiber, SC |

## END OF SECTION

DIVISION 27 21 29

CNGE2MC-M SERIES – FIBER OPTIC GIGABIT ETHERNET TRANSCEIVER

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- Fiber Optic Ethernet Transceiver

1.02 SECTION INCLUDES

- CNGE2MC-M Series Gigabit Ethernet Transceiver

1.03 REFERENCES

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide an Ethernet transceiver system that transmits bi-directional 10, 100, or 1000Mps data over multimode or single mode fiber connections.

- The system shall utilize CNGE2MC-M media converter package along with an SFP modules capable of bi-directional data transmission of 10, 100, or 1000Mbps on one or two optical fibers, depending on the SFP module selected. (See Table C, page 3.)

1.05 SUBMITTALS

- Product Data: Manufacturer’s printed product data sheet for each type of Transmitter/Receiver specified.

- Detail Drawings: Electrical and optical connect drawings. Product mounting template.

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for each type of Transmitter/Receiver specified.

- Warranty: Manufacturer’s Printed Warranty

1.06 DELIVERY, STORAGE AND HANDLING

- Deliver materials in unopened factory packaging with Manufacturer’s bar coding to the job site.

- Inspect product upon delivery to assure that specified products have been received.

- Store in original packaging in a climate controlled environment. Storage Temperature not to exceed: -40˚ C to +85˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of –40˚ C to +75˚ C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 0% to 95% (non-condensing). If product is installed in condensation conditions, unit shall have conformal coating applied to the printed circuit board.

1.08 WARRANTY

- Standard Communication Networks Comprehensive Lifetime Warranty: Comnet warrants the product to be free of factory defects under manufacture’s Lifetime Warranty as submitted under article 1.05 (E)

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer: Communication Networks 3 Corporate Drive Danbury, CT USA; Telephone: 203-796-5300; Fax 203-796-5303; Email: sales@comnet.net; Internet: www.comnet.net

- Substitutions: Not Permitted

- All fiber optic modules shall be supplied from a single manufacturer.

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table C: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Ethernet Gigabit data transceiver system shall consist of a Comnet CNGE2MC-M series media converter and a Comnet SFP module.  The module shall support the transmission of 10, 100, or 1000 Mbps employing singlemode or multimode optical fiber. The module shall support the Ethernet data interface and be IEEE 802.3 compliant.  The unit is set to auto-negotiate data rates and can be used for half duplex or full duplex transmission.  The module shall require no in-field optical adjustments or in-line attenuators to ease installation.  The module shall provide power and fiber link status indicating LED’s for monitoring proper system operation. The modules shall provide automatic re-settable solid-state current limiters and independent voltage regulators on each module to reduce the chance of a single point failure of the system. The module shall have an MTBF of >100,000 hours and operate in an environment of –40˚ C to +75˚ C and relative humidity between  0% to 95% (non-condensing). The module shall be UL and ULC listed and CE marked. The circuit board shall be UL 94 flame rated and meet all PCI standards.  All PC boards shall be designated with part number, PC board number and show appropriate revision number. Housing shall be of all metal construction.  All LED indicators and both electrical and mechanical connections shall be identified with silk-screened labels. The module shall have a lifetime warranty to reduce system life cycle cost in an event of a module failure.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3

- Data Rate: 10/100/1000 Mbps

- Operation Mode: Full or Half Duplex

2.05 OPTICAL SPECIFICATIONS

- Determined by SFP selection – see Table C, page 3.

2.06 STATUS INDICATORS

- Power: On/Green – Off/Off

- Fiber Link:  Fiber connection/Green – No Fiber Connection/Red

2.07 CONNECTORS

- Determined by SFP module selected – see Table C, page 3.

- Power: Screw Clamp Terminal Strip.

- Data: RJ-45

2.08 ELECTRICAL SPECIFICATIONS

- Power: Surface Mount:

12VDC @ 200 mA

- Current Protection: Automatic re-settable solid-state current limiters

- Voltage Regulation: Solid-state, Independent on each board

- Circuit Board: UL 94 flame rated and meets all IPC standards.

2.09 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 3.3” x 2.5” x 1.1” (8.4 cm x 6.4 cm x 2.8 cm)

- Finish: Module shall be constructed of a metal enclosure with all connections and indicators silk-screened directly on unit.

- Weight: <2.0 lbs./0.9kg

2.10 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >100,000 Hours

- Operating Temp: –40˚ C to +75˚ C

- Storage Temp: -40˚ C to +85˚ C

- Relative Humidity: 0% to 95% (non-condensing). If product is installed under condensation conditions, unit shall have conformal coating applied to the printed circuit board. (Add –C to model number for conformal coated printed circuit board)

2.11 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- UL 94-flame rated PCB board

- European Listed (CE)

PART 3 - EXECUTION

3.01 EXAMINATION

- Inspect modules before installation.

- Modules shall be free of any cosmetic defects or damage.

- All optical connectors shall be covered with dust caps and remain on the module until installing cable connectors to module.

- Shipping box shall include the module, power supply and operations manual.

3.02 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the ComNet mounting template and installation manual.

- Optical Fibers

- Caution: NEVER look into the end of an active optical fiber when using laser light output. Eye damage can occur. Wear eye protection when cleaving, terminating, and splicing fiber.

- The number and type (multimode or single-mode) of optical fiber shall meet the requirements of the ComNet model number in article 2.05 used in the installation.

- All optical fiber cables shall be properly installed and terminated with the mating optical connectors as submitted in article 2.07 (A).

- The optical link shall be tested with either a power meter, at a minimum, or OTDR to ensure the link budget (overall path loss) plus an added 3dB of optical safety margin does not exceed the optical power budget as submitted in article 2.05.

- All optical connectors on cable shall be cleaned in compliance to optical connector manufactures specifications and covered with dust caps until connection to the fiber optic module.

3.03 INSTALLATION

- General: Locate fiber optic modules as indicated on the approved detail drawings and install module in compliance with the ComNet installation and operations manual.

3.04 TESTING

- Testing the Fiber Optic Ethernet Link.

- Verify that the data leads and optical fibers are properly connected.

- Make sure that power is applied to all fiber optic modules, controllers, and receiver drivers or other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all fiber scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| CNGE2MC-M SERIES | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| CNGE2MC-M | 10/100/1000 Mbps Ethernet Transceiver | SFP-dependant (see Table C) |

Table B: Product Compatibility Chart

| COMPATIBLE TRANSCEIVER | DESCRIPTION |
| --- | --- |
| CNGE2MC | 10/100/1000 Mbps Ethernet Transceiver, Standard Size |
| CNGE2MC-M | 10/100/1000 Mbps Ethernet Transceiver, Minature Size |
| CNGE22MC | Dual 10/100/1000 Mbps Ethernet Transceiver, Standard Size |
| CNGE4US | 4-port 10/100/1000 Mbps Unmanaged Switch |
| CNGE8US | 8-port 10/100/1000 Mbps Unmanaged Switch |
| CNGE3FE7MS | 10-port Managed Switch, (7) 10/100 RJ-45 ports, (3) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |
| CNGE3FE7MS-2 | 10-port Managed Switch, (7) 10/100 RJ-45 ports, (3) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |
| CNGE2FE24MS | 26-port Managed Switch, (24) 10/100 RJ-45 ports, (2) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |
| CNGE2FE24MS-POE | 26-port Managed Switch, (24) 10/100 RJ-45 802.3af PoE ports, (2) combo ports – 10/100/1000 Mbps RJ-45 ports and 1000Base FX SFP ports |

Table C: SFP Module Compatibility Chart

| SFP | DESCRIPTION |
| --- | --- |
| SFP-6 | Single Mode, 1000fx, 1310nm, 15km, 2 Fiber, LC |
| SFP-7 | Single Mode, 1000fx, 1310nm, 40km, 2 Fiber, LC |
| SFP-8 | Single Mode, 1000fx,1550nm, 70km, 2 Fiber, LC |
| SFP-9 | Single Mode, 1000fx,1550nm, 120km, 2 Fiber, LC |
| SFP-12 | Single Mode, 1000fx, 1310nm, 20km, 1 Fiber, LC |
| SFP-13 | Single Mode, 1000fx, 1550nm, 20km, 1 Fiber, LC |
| SFP-14 | Single Mode, 1000fx, 1310nm, 20km, 1 Fiber, SC |
| SFP-15 | Single Mode, 1000fx, 1550nm, 20km, 1 Fiber, SC |
| SFP-16 | Multimode, 1000fx, 850 nm 550m, 2 Fiber, LC |
| SFP-17 | Single Mode, 1000fx, 1310nm, 60km, 1 Fiber, LC |
| SFP-18 | Single Mode, 1000fx,1550nm,60km, 1 Fiber, LC |
| SFP-21 | Single Mode, 1000fx, 1310nm, 60km, 1 Fiber, SC |
| SFP-22 | Single Mode, 1000fx,1550nm,60km, 1 Fiber, SC |

## END OF SECTION

DIVISION 27 21 29

CNGE22MC SERIES – DUAL FIBER OPTIC GIGABIT ETHERNET TRANSCEIVER

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- Dual Fiber Optic Gigabit Ethernet Transceiver

1.02 SECTION INCLUDES

- CNGE22MC Series Dual Gigabit Ethernet Transceiver

1.03 REFERENCES

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide a dual Ethernet transceiver system that transmits bi-directional 10, 100, or 1000Mps data over multimode or single mode fiber connections.

- The system shall utilize CNGE22MC media converter package along with SFP modules capable of two channels of bi-directional data transmission of 10, 100, or 1000Mbps on one or two optical fibers, depending on the SFP module selected. (See Table C, page 3.)

1.05 SUBMITTALS

- Product Data: Manufacturer’s printed product data sheet for each type of Transmitter/Receiver specified.

- Detail Drawings: Electrical and optical connect drawings. Product mounting template.

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for each type of Transmitter/Receiver specified.

- Warranty: Manufacturer’s Printed Warranty

1.06 DELIVERY, STORAGE AND HANDLING

- Deliver materials in unopened factory packaging with Manufacturer’s bar coding to the job site.

- Inspect product upon delivery to assure that specified products have been received.

- Store in original packaging in a climate controlled environment. Storage Temperature not to exceed: -40˚ C to +85˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of –40˚ C to +75˚ C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 0% to 95% (non-condensing). If product is installed in condensation conditions, unit shall have conformal coating applied to the printed circuit board.

1.08 WARRANTY

- Standard Communication Networks Comprehensive Lifetime Warranty: Comnet warrants the product to be free of factory defects under manufacture’s Lifetime Warranty as submitted under article 1.05 (E)

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer: Communication Networks 3 Corporate Drive Danbury, CT USA; Telephone: 203-796-5300; Fax 203-796-5303; Email: sales@comnet.net; Internet: www.comnet.net

- Substitutions: Not Permitted

- All fiber optic modules shall be supplied from a single manufacturer.

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table C: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Gigabit Ethernet data transceiver system shall consist of a Comnet CNGE22MC series dual media converter and Comnet SFP modules.  The module shall support the transmission of tow channels of 10, 100, or 1000Mbps employing singlemode or multimode optical fiber. The module shall support the Ethernet data interfaces and be IEEE 802.3 compliant.  The unit is set to auto-negotiate data rates and can be used for half duplex or full duplex transmission.  The module shall require no in-field optical adjustments or in-line attenuators to ease installation.  The module shall provide power and fiber link status indicating LED’s for monitoring proper system operation. The modules shall provide automatic re-settable solid-state current limiters and independent voltage regulators on each module to reduce the chance of a single point failure of the system. The module shall have an MTBF of >100,000 hours and operate in an environment of –40˚ C to +75˚ C and relative humidity between  0% to 95% (non-condensing). The module shall be UL and ULC listed and CE marked. The circuit board shall be UL 94 flame rated and meet all PCI standards.  All PC boards shall be designated with part number, PC board number and show appropriate revision number. Housing shall be of all metal construction.  All LED indicators and both electrical and mechanical connections shall be identified with silk-screened labels. The module shall have a lifetime warranty to reduce system life cycle cost in an event of a module failure.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3

- Data Rate: 10/100/1000 Mbps

- Operation Mode: Full or Half Duplex

2.05 OPTICAL SPECIFICATIONS

- Determined by SFP selection – see Table C, page 3.

2.06 STATUS INDICATORS

- Power: On/Green – Off/Off

- Fiber Link:  Fiber connection/Green – No Fiber Connection/Red

2.07 CONNECTORS

- Determined by SFP module selected – see Table C, page 3.

- Power: Screw Clamp Terminal Strip.

- Data: RJ-45

2.08 ELECTRICAL SPECIFICATIONS

- Power: Surface Mount:

12VDC @ 200 mA

- Current Protection: Automatic re-settable solid-state current limiters

- Voltage Regulation: Solid-state, Independent on each board

- Circuit Board: UL 94 flame rated and meets all IPC standards.

2.09 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 6.1” x 5.3” x 1.1” (15.5 cm x 13.5 cm x 2.8 cm)

- Finish: Module shall be constructed of a metal enclosure with all connections and indicators silk-screened directly on unit.

- Weight: <2.0 lbs./0.9kg

2.10 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >100,000 Hours

- Operating Temp: –40˚ C to +75˚ C

- Storage Temp: -40˚ C to +85˚ C

- Relative Humidity: 0% to 95% (non-condensing). If product is installed under condensation conditions, unit shall have conformal coating applied to the printed circuit board. (Add –C to model number for conformal coated printed circuit board)

2.11 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- UL 94-flame rated PCB board

- European Listed (CE)

PART 3 - EXECUTION

3.01 EXAMINATION

- Inspect modules before installation.

- Modules shall be free of any cosmetic defects or damage.

- All optical connectors shall be covered with dust caps and remain on the module until installing cable connectors to module.

- Shipping box shall include the module, power supply and operations manual.

3.02 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the ComNet mounting template and installation manual.

- Optical Fibers

- Caution: NEVER look into the end of an active optical fiber when using laser light output. Eye damage can occur. Wear eye protection when cleaving, terminating, and splicing fiber.

- The number and type (multimode or single-mode) of optical fiber shall meet the requirements of the ComNet model number in article 2.05 used in the installation.

- All optical fiber cables shall be properly installed and terminated with the mating optical connectors as submitted in article 2.07 (A).

- The optical link shall be tested with either a power meter, at a minimum, or OTDR to ensure the link budget (overall path loss) plus an added 3dB of optical safety margin does not exceed the optical power budget as submitted in article 2.05.

- All optical connectors on cable shall be cleaned in compliance to optical connector manufactures specifications and covered with dust caps until connection to the fiber optic module.

3.03 INSTALLATION

- General: Locate fiber optic modules as indicated on the approved detail drawings and install module in compliance with the ComNet installation and operations manual.

3.04 TESTING

- Testing the Fiber Optic Ethernet Link.

- Verify that the data leads and optical fibers are properly connected.

- Make sure that power is applied to all fiber optic modules, controllers, and receiver drivers or other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all fiber scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| CNGE22MC SERIES | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| CNGE22MC | 10/100/1000 Mbps Gigabit Ethernet Transceiver | SFP-dependant (see Table C) |

Table B: Product Compatibility Chart

| COMPATIBLE TRANSCEIVER | DESCRIPTION |
| --- | --- |
| CNGE2MC | 10/100/1000 Mbps Ethernet Transceiver, Standard Size |
| CNGE2MC-M | 10/100/1000 Mbps Ethernet Transceiver, Minature Size |
| CNGE22MC | Dual 10/100/1000 Mbps Ethernet Transceiver, Standard Size |
| CNGE4US | 4-port 10/100/1000 Mbps Unmanaged Switch |
| CNGE8US | 4-port 10/100/1000 Mbps Unmanaged Switch |
| CNGE3FE7MS | 10-port Managed Switch, (7) 10/100 RJ-45 ports, (3) combo ports |
| CNGE3FE7MS-2 | 10-port Managed Switch, (7) 10/100 RJ-45 ports, (3) combo ports |
| CNGE2FE24MS | 26-port Managed Switch, (24) 10/100 RJ-45 ports, (2) combo ports |
| CNGE2FE24MS-POE | 26-port Managed Switch, (24) 10/100 RJ-45 802.3af PoE ports, (2) combo ports |

Table C: SFP Module Compatibility Chart

| SFP | DESCRIPTION |
| --- | --- |
| SFP-1 | Copper, 10/100/1000TX, 100 m, RJ-45 |
| SFP-6 | Single Mode, 1000FX, 1310nm, 15km, 2 Fiber, LC |
| SFP-7 | Single Mode, 1000FX,1310 nm, 40km, 2 Fiber, LC |
| SFP-8 | Single Mode, 1000FX,1550 nm, 70km, 2 Fiber, LC |
| SFP-9 | Single Mode, 1000FX, 1550 nm, 120km, 2 Fiber, LC |
| SFP-12 | Single Mode, 1000FX, 1310/1550 nm, 20km, 1 Fiber, LC |
| SFP-13 | Single Mode, 1000FX, 1550/1310 nm, 20km, 1 Fiber, SC |
| SFP-14 | Single Mode, 1000FX, 1310/1550 nm, 20km, 1 Fiber, SC |
| SFP-15 | Single Mode, 1000FX, 1550/1310 nm 20km, 1 Fiber, SC |
| SFP-16 | Multimode, 1000FX, 850nm, 300 m, 2 Fiber, SC |
| SFP-17 | Single Mode, 1000FX,1310/1550nm, 60km, 1 Fiber, LC |
| SFP-18 | Single Mode, 1000FX, 1550/1310nm, 60km, 1 Fiber, LC |
| SFP-21 | Single Mode, 1000FX,1310/1550nm, 60km, 1 Fiber, SC |
| SFP-22 | Single Mode, 1000FX, 1550/1310nm, 60km, 1 Fiber, SC |

## END OF SECTION