DIVISION 27 21 29

CWGE2FE24MODMS –24-PORT ETHERNET LAYER 2 MANAGED SWITCH

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- Twenty-four Port Ethernet Layer 2 Managed Switch

1.02 SECTION INCLUDES

- CWGE2FE24MODMS Series Gigabit Ethernet Layer 2 Managed Switch and Plug-in Modules

1.03 REFERENCES

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- European Union Compliance (CE)

- NEMA TS2-1998

- CALTRANS

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide an Ethernet Layer 2 Managed Switch that is capable of accepting up to three 8-port modules.  Modules shall be available with eight 10/100 RJ-45 ports and eight 100FX SFP ports.  The switch shall have two Gigabit combo aggregate or uplink ports.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for each type of device specified.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment. Storage Temperature not to exceed -40˚ C to +70˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of 0˚ C to +45˚ C.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 5% to 95% (non-condensing).

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer: Communication Networks; 3 Corporate Drive; Danbury CT 06810 USA; Telephone: 203-796-5300; Fax 203-796-5303; Email: sales@comnet.net; Internet: www.comnet.net

- Substitutions: Not Permitted

- All Ethernet managed switches and SFP modules shall be supplied from a single manufacturer.

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table B: Product Compatibility Chart

  - GENERAL SPECIFICATIONS

- The Ethernet managed switch system shall be a Comnet CWGE2FE24MODMS-Series system.  The switch shall support transmission utilizing Category 5 cable or better, multimode, or single-mode fiber.  The switch shall support IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.  The switch shall feature three bays.  Each bay shall be capable of accepting one of the following modules: Modules shall be available with eight 10/100 RJ-45 ports and eight 100FX SFP ports.  The switch shall have two Gigabit combo aggregate or uplink ports.  The switch and modules shall require no in-field electrical or optical adjustments or in-line attenuators to ease installation. The module shall have redundant power supply connections to minimize single point failure.  The module shall have a five-year warranty.

- The following IEEE Networking Standards shall be supported:

IEEE 802.3 10Base-T Ethernet

IEEE 802.3u 100Base-TX Fast Ethernet

IEEE 802.3ab 1000Base-TX

IEEE 802.3z Gigabit Ethernet Fiber

IEEE 802.3x Flow Control and Back-pressure

IEEE 802.1p class of service

IEEE 802.1Q VLAN and GVRP

IEEE 802.1D-2004 Rapid Spanning Tree Protocol (RSTP)

IEEE802.3ad LACP

IEEE802.1X Port_based Network Access Control

- Switching Performance

Switch Technology: Store and Forward Technology with 32Gbps Switch Fabric.

System Throughput: 14,880pps for 10M Ethernet; 148,800pps for 100M Fast Ethernet; 1,488,100 for Gigabit Ethernet

Transfer Packet Size: 64 bytes to 1522 bytes (with VLAN Tag)

MAC Address: 8K MAC

Packet Buffer: 1Mbits

Relay Alarm: Dry Relay output with 1A@24V ability

- MANAGEMENT

Configuration: Web, HTTPS, SSH,TFTP/Web Update for firmware and configuration backup/restore, DHCP Client, Warm reboot, Reset to

default, Admin password, Port Speed/Duplex control, status, statistic, MAC address table display, Static MAC, Aging time, SNMP v1, v2c, v3, Traps and RMON1.

SNMP MIB: MIB-II, Bridge MIB, VLAN MIB, SNMP MIB, RMON and Private MIB

Port Trunk: Up to 5 Static Trunk and 802.3ad LACP

VLAN: IEEE802.1Q VLAN, GVRP. Up to 64 VLAN groups

Quality of Service: Four priority queues per port,

IEEE802.1p COS and Layer 3 TOS/DiffServ

IGMP Snooping: IGMP Snooping V1/V2 for multicast filtering and IGMP Query

Rate Control: Ingress filtering for Broadcast, Multicast, Unknown DA or all packets, and Egress filtering for all packets

NTP: Network Time Protocol to synchronize time from Internet

Embedded Watchdog: Embedded hardware watchdog

timer to auto reset system when switch system failure

Port Mirroring: Online traffic monitoring on multiple selected ports

Port Security: Assign authorized MAC to specific port

IP Security: IP security to prevent unauthorized access

802.1x: Port_based Network Access Control

DHCP Server: Can assign 255 IP address, support IP and MAC binding

System Log: Supports both Local mode and Server mode

NETWORK REDUNDANCY

Rapid Spanning Tree Protocol: IEEE802.1D-2004 Rapid Spanning Tree Protocol.

Compatible with Legacy STP and IEEE802.1w.

Rapid Super Ring (RSR): Ring Redundancy Technology.

Failure recovery within 5ms.

Dual Homing II: Multiple uplink paths to upper switches

Multiple Ring: Couple or multiple Rapid Super Rings

Legacy Super Ring: Backward compatible in client mode

Any Ring: Inter-operate with other vendors' ring Interface

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3

- Data Rate: up to 100 Mbps (1000 Mbps via uplink ports)

- Data Inputs/Outputs: up to 24 + 2

- Operation Mode: Half or Full Duplex

2.06 STATUS INDICATORS

- Power: Proper Power = Green

- RJ-45 Link/Data: Green,  No Link/No Data: Off

- SFP Link/Data: Green, No Link/No Data: Off

2.07 CONNECTORS

- Optical: LC or SC, SFP-dependant

- Power:  Screw Clamp Terminal Strip.

- Data: RJ-45

2.08 ELECTRICAL SPECIFICATIONS

- Power:

AC 100 V to 240 V or DC 12 V to 48 V

- Connection for redundant power shall be available.

2.09 MECHANICAL SPECIFICATIONS

- 17.3 in (W) x 11.0 in (D) x 1.7 in (H)		440 mm (W) x 280 mm (D) x 44 mm (H)

- Finish: Module shall be constructed of a metal enclosure with a powder coat.

- Weight: <2 lb/1kg

2.10 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >100,000 Hours

- Operating Temp: 0˚ C to +45˚ C

- Storage Temp: -40˚ C to +70˚ C

- Relative Humidity: 5% to 95% (non-condensing).

2.11 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Underwriters Laboratory (UL) Listing Number: I.T.E. 6D16

- Underwriters Laboratory Canada (ULC) Listing Number: I.T.E. 6D16

- UL 94-flame rated PCB board: 94VO

PART 3 - EXECUTION

3.01 EXAMINATION

- All optical connectors shall be covered with dust caps and remain on the module until installing cable connectors to module.

3.02 PREPARATION

- Optical Fibers

- Caution: NEVER look into the end of an active optical fiber when using laser light output. Eye damage can occur. Wear eye protection when cleaving, terminating, and splicing fiber.

- The number and type of optical fiber shall meet the requirements of the Comnet model number in article 2.05 used in the installation.

- All optical fiber cables shall be properly installed and terminated with the mating optical connectors as submitted in article 2.07 (A).

- The optical link shall be tested with either a power meter, at a minimum, or OTDR to ensure the link budget (overall path loss) plus an added 3dB of optical safety margin does not exceed the optical power budget as submitted in article 2.05.

- All optical connectors on cable shall be cleaned in compliance to optical connector manufactures specifications and covered with dust caps until connection to the fiber optic module.

3.03 INSTALLATION

- General: Locate switch as indicated on the approved detail drawings and install modules in compliance with the Comnet installation and operations manual.

3.04 TESTING

- Testing the Ethernet Equipment.

- Verify that data and any optical fibers are properly connected.

- Make sure that power is applied to the switch and tp the other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all fiber scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| CWGE2FE24MODMS SERIES | DESCRIPTION |
| --- | --- |
| CWGE2FE24MODMS/CHASSIS | Layer 2 Switch chassis |
| CWGE2FE24MOD/8FXSFP | Module with eight 100FX SFP ports |
| CWGE2FE24MOD/8TX | Module with eight 10/100 RJ-45 ports |

Table B: Product Compatibility Chart

Compatible SFP transceiver modules

SFP-1

SFP-6

SFP-7

SFP-8

SFP-9

SFP-12A

SFP-12B

SFP-14A

SFP-14B

SFP-16

SFP-18A

SFP-18B

SFP-22A

SFP-22B

## END OF SECTION