DIVISION 27 21 29

PS-9VDC

SWITCHING MODE POWER SUPPLY

ENGINEERING SPECIFICATIONS

PART 1 - GENERAL

- SUMMARY

  - Switching Mode Power Supply

- SECTION INCLUDES

  - PS-9VDC

- REFERENCES

  - Underwriters Laboratory (UL)

  - Underwriters Laboratory Canada (ULC)

  - European Union Compliance (CE)

  - Federal Communications Commission (FCC)

- SYSTEM DESCRIPTION

  - Performance Requirements: Provide a power supply to power modules.

- SUBMITTALS

  - Manufacturer’s Installation and Operating Manual: Printed installation and operating information for each type of device specified.

- DELIVERY, STORAGE AND HANDLING

  - Store in original packaging in a climate controlled environment.  Storage Temperature not to exceed -25˚ C to +85˚ C

- PROJECT/SITE CONDITIONS

  - Temperature Requirements: Products shall operate in an environment with an ambient temperature range of 0.0˚ C to +40˚ C without the assistance of fan-forced cooling.

  - Humidity Requirements: Products shall operate in an environment with relative humidity of 20% to 95% (non-condensing).

PART 2 - PRODUCTS

- MANUFACTURER

  - Acceptable Manufacturer: Communication Networks; 3 Corporate Drive; Danbury CT 06810 USA; Telephone: 203-796-5300; Fax 203-796-5303; Email: sales@ComNet.net; Internet: www.ComNet.net

  - Substitutions: Not Permitted

- GENERAL SPECIFICATIONS

  - The power supply shall be a switching mode power supply (SMPS).

  - The SMPS shall take 100 - 240VAC, Class II double insulated @ 50 to 60 Hz @ 700mA.

  - The SMPS shall produce a stable 9 VDC and be capable of supplying 0.7 amperes at 9 VDC (6.3 W).

- STATUS INDICATORS

  - N/A

- CONNECTORS

  - Power Connections: Flying Leads

- MECHANICAL SPECIFICATIONS;

- PS-9VDC-US: 2.9 x 1.9 x 1.1 in.,

(7.4 x 4.8 x 2.8 cm)

- PS-9VDC-EU:2.63 x 1.38 x 1.63 in.,

(6.67 x 3.49 x 4.13 cm)

- Weight: <0.5 lb./0.25 kg

- ENVIRONMENTAL SPECIFICATIONS

  - Operating Temp: 0.0˚ C to +50˚ C

  - Storage Temp: -25˚ C to +85˚ C

  - Relative Humidity: 20% to 95% (non-condensing).

  - Cooling: Natural convection

- APPROVALS AND EMC COMPLIANCE

- UL1310 Class 2

- UL508 Listed

- UL60950-1

- EN60950-1

- CE

- EN55022 Class A

- RoHS Compliant

PART 3 - EXECUTION

- EXAMINATION

  - The SMPS exterior should be free from dents or other damage that would indicate a compromise in performance or safety.

  - The DIN rail clip should be in good condition and solidly connected to the switch.

  - Ventilation holes shall be in good condition and free from debris to allow natural convection to occur.

  - Screw terminals shall be in good condition.

- MOUNTING

  - The SMPS switch shall be mounted such that its chassis is at least 25 mm (1 in) on all sides from any other objects to allow adequate cooling to occur.